KENO sculpture · Ōtaki Beach, Wellington, New Zealand · kenosculpture.co.nz

OUTDOOR SCULPTURE CARE

Taking care of your outdoor sculpture

Oamaru limestone is a durable stone — but outdoors it is exposed to rain, moisture and mould. Two simple routines keep your piece looking its best for generations.

ALREADY DONE FOR YOU

Your sculpture arrives sealed and ready to display

- Every piece is professionally sealed before it leaves the studio — no prep required when it arrives.

- Oamaru limestone weathers beautifully. Expect the stone to develop a gentle patina over time — this is natural and adds character.

- Questions about your specific piece? Brett is always happy to advise on care for your environment and climate.

RECOMMENDED CARE CYCLE · EVERY 6–12 MONTHS

Mould & moss treatment

Mould and moss will naturally grow on outdoor stone over time. This simple four-step routine removes it and keeps your sculpture looking its best.

01 Apply the treatment Mix one of the following with water and apply evenly to all surfaces. Spray or brush on, working from top to bottom: - 30 Seconds — a concentrated mould and mildew remover - Janola — diluted household bleach - Wet & Forget — a spray-and-leave mould inhibitor Use a brush for textured areas and hollows where spray may not reach. International clients: these are New Zealand brands. Look for a diluted bleach solution or a mould-and-mildew treatment suitable for stone surfaces.

02 Let it work Leave the treatment on for 15 minutes. Keep surfaces wet throughout — reapply spray 2–3 times during this period if needed. If mould or moss is stubborn, scrub with a firm brush before rinsing. A second treatment may be needed for heavy build-up. FOR VERY STUBBORN MOULD ONLY Sand lightly with 120 grit sandpaper. Dust off thoroughly before retreating. Use this step sparingly — it is not part of the regular routine.

03 Rinse thoroughly Rinse the sculpture thoroughly with a hose, working top to bottom. Make sure all treatment residue is washed away completely.

04 Allow to dry fully Allow the stone to dry completely in natural air. Do not rush this — the stone must be fully dry before rain or before moving back to its position. That is the 6–12 month cycle complete. Your sculpture is clean and ready for another season outdoors.

FULL RESEAL · EVERY 2–3 YEARS

Resealing your sculpture

In addition to the regular mould treatment, your sculpture needs a full reseal every 2–3 years. This refreshes the protective layer and keeps moisture out of the stone.

01 Make sure the stone is dry The stone must be completely dry before sealing — any moisture trapped beneath the sealer will cause problems. Allow at least 48 hours of dry weather before you begin. Do not seal in direct sunlight or on a hot day.

02 Apply the sealer Apply a penetrating masonry sealer liberally, starting from the top and working down. Use a brush or roller for even coverage. In New Zealand: KiwiCare No Leaks Masonry Sealer or Bone Dry by Wet & Forget. Internationally: look for a penetrating stone or masonry sealer — not a surface coating. The sealer should soak into the stone, not sit on top of it. Christine can supply a 200ml bottle of KiwiCare No Leaks Masonry Sealer for $25 NZD — enough for most sculptures. Contact kenosculpture@gmail.com to order.

03 Don't forget the base Seal the bottom of the sculpture if you can — moisture enters from below as well as above. Tilt the piece carefully and apply sealer to the base and lower edges. This makes a real difference to long-term durability, particularly in wetter climates.

04 Leave it to cure Any streaks or marks will fade as the sealer dries and cures — do not worry if it looks uneven while wet. Allow the sealer to dry completely before rain or moisture. The reseal is complete — your sculpture is protected for another 2–3 years.

Not sure which products suit your climate? Brett and Christine are happy to advise.

kenosculpture@gmail.com  ·  021 855 088  ·  kenosculpture.co.nz

Please sign off all care instructions with Brett before publishing.